HUMAN-TO-ROBOT HANDOVER BENCHMARK

| Reference No / Version | P-H2RHO-0.1 |
| --- | --- |
| Authors | Ricardo Sanchez-Matilla, Konstantinos Chatzilygeroudis, Apostolos Modas, Nuno Ferreira Duarte, Alessio Xompero, Pascal Frossard, Aude Billard, and Andrea Cavallaro |
| Institution | Centre of Intelligent Sensing - Queen Mary University of London, UK; LASA&LTS, Swiss Federal School of Technology in Lausanne, Switzerland; VisLab, Institute of Systems and Robotics, Lisbon, Portugal |
| Contact information | ricardo.sanchezmatilla@qmul.ac.uk, konstantinos.chatzilygeroudis@epfl.ch |
| Adopted Protocol | Human-to-robot handover protocol (P-H2RHO-0.1) |
| Scoring | Fill the attached table with the following rules for all the configurations: - Fill the cup with the instructed amounts of content (empty and 90%) - Weight filling before and after the execution of the handover - Measure distance between the initial (e.g. center of the table) and the delivery location of the cup - Annotate elapsed time for the different phases: human maneuvering, handover, robot maneuvering (e.g. through visual inspection on recorded videos or automatic algorithm) - Annotate estimated measures: width at the top of cup, width at the bottom, height, mass (cup + filling), fullness, delivery location, and mass of the delivered content. - Annotate human-intention and end-effector with provided pre-recorded data |
| Details of Setup | See human-to-robot handover protocol |
| Results to Submit | Submission to our website (to be provided) - Scoring table (attached to the benchmark) - Detailed comments on: - Videos of all executed configurations - Details of the hardware setup (robot and sensing) - Any artificial illumination used. |